# Generatre Synthetic Data (Fv, T, RGB) Algorithm

Mosheat Alon-Moses, May 2025

Contents

Introduction	2

Generate Fv, T Data	3

Flame heights	3

Fv Vectors	3

T vectors	4

Fv Matrix	5

T Matrix	6

Generate RGB Image	7

sootCalculation.mat	7

Run through SootImage	7

Results	8

Known Issues and Future Work	10

Appendix A – Accepting Data Restrictions	11

Appendix B – Generate Data (Matlab Code)	12

Generate Fv	12

Generate T	12

Appendix C – SootImage (Matlab Code)	12

# Introduction

Our research relies on flame RGB images for investigating soot properties. My role is to create a deep learning model for extracting the soot properties upon a given RGB digital camera image of an axisymmetric laminar flame. For me to train such a model, I need data. Existing simulators have their downside as they are slow, not very accurate and complicated for configuration. There was a big need for creating such synthetic data. 
Victor created a few Matlab files calculating properties of 3 vectors of the flame. After filtering out problematic data- based on a linear interpolation, I completed values of Fv (soot fractions volume) and T (temperature) in each pixel of the flame. I then used Victor’s sootImage for creating an RGB image of the flame upon the given generated data.

# Generate Fv, T Data

Generating the data is done with a few random picks. Therefore, there are a few restrictions that must be verified before the data generated is accepted for next steps.
Once the two heights (h_top, h_maxFv), three Fv vectors (fbase, fmaxFv, ftop) and three T vectors (tbase, tmaxFv, ttop) are generated as detailed below- the data goes through a number of verification steps as detailed in Appendix A. If verification fails- the data is regenerated until all terms and restrictions are met.

## Flame heights

To generate the flame’s contour, I begin with randomly generating  2 heights:

- h_top - the top (currently, it isn’t the tip of the flame, but it is the top height of the flames we produce)

- h_maxFv – this is a height that holds the maximum Fv (soot fractions volume) value of the flame

h_maxFv = random.randint(200,400) #random height for max fv

h_top = random.randint(200,400) + h_maxFv #random height for top flame

First, I draw a random number between 200 and 400 as the flame’s h_maxFv.
Next, I draw another random number between 200 and 400 and add it to h_maxFv. This is the flame’s h_top.

When relating to the base of the flame, height 0.

## Fv Vectors

Once I have h_maxFv and h_top heights, I run Matlab functions to generate 3 vectors of soot fractions volume in every height (base, maxFv, top):

fbase = generatef(1)
fmaxFv = generatef(4)
ftop = generatef(7)

Now I know the size of each vector in every height, I can create the flame’s Fv contour.

The Fv contour will be the flame’s edges I will regard to when dealing with both the soot fractions volume and the temperature properties.

Notice fmaxFv holds the highest Fv value and len(fbase)>len(fmaxFv)>len(ftop). Both restrictions are by definition.

See Appendix A for all restrictions.

## T vectors

Next, I generate the temperature vectors. Here too I regard h_maxFv and h_top heights. I run Matlab functions to generate 3 vectors of temperature in every height (base, maxFv, top):

tbase = generateT(1)
tmaxFv = generateT(4) 
ttop = generateT(7)

For some reason, the T vectors are generated vertically, so I transpose them before continuing.
Flame’s T contour is different than flame’s Fv contour. It is generally wider, and there is no rule as for which section (ttop, tmaxFv or tbase) is wider. 
In any case- populating tmatrix is done according to flame’s Fv contour.

See Appendix A for all restrictions for accepting the generated data.

## Fv Matrix

After generating the 3 Fv vectors, I want to populate the Fv matrix. I run on each (x,y) in the Fv matrix (fvmat).

- Create a matrix fvmat with shape of:

width = len(fbase) + 50

height = h_top + 10

- Initialize fvmat with zeros and attend only the cells within the Fv contour. This way- values outside the contour remain 0.

- Insert fbase at y=0, fmaxFv at y=h_maxFv and ftop at y=h_top.

- For cells on the Fv contour submit the value 0.1.

- Fill in the missing data inside the Fv contour boundaries by calculating a linear interpolation between the bottom vector and the top vector, depending on the weight calculated according to the Y position. This means when weight is 0, I’ll get values as bottom_vector[x]; when weight is 1, I’ll get values as top_vector [x]; and for values in between, I’ll get a blend. By doing this I get smooth transitions from the base to the top as I move vertically.
This is done in two regions- one between fbase and fmaxFv and the other one between fmaxFv and ftop. 
In each region I calculate:

- weight = (y - 0) / (h_maxFv-0)

- Because the bottom vector is longer than the top vector, I have 2 cases-

  - If x is within the range of top vector:
Interpolate between bottom_vector[x] and  top_vector [x] using the formula:
(1 - weight) * bottom_vector[x] + weight * top_vector[x]

  - If x is outside the range of top_vector (so top_vector[x] does not exist), and the current matrix value is 0:
Interpolate between base[x] and a small constant value of 0.1, using the same weight:
(1 - weight) * bottom_vector[x] + weight *0.1

## T Matrix

After generating the 3 T vectors, I want to populate the T matrix. I run on each (x,y) in the T matrix.

- Create a matrix tmat with shape of:

width = max(len(tbase), len(tmaxFv), len(ttop)) + 50

height = h_top + 10

- Initialize tmat with 300 (units are in Kalvin, so this is the zero value) and attend only the cells within the Fv contour. This way- values outside the contour remain 300.
* Reminder- populating tmatrix is done according to flame’s Fv contour.

- Insert tbase at y=0, tmaxFv at y=h_maxFv and ttop at y=h_top.

- For cells on the Fv contour submit the value 300.

- Fill in the missing data inside the Fv contour boundaries by calculating a linear interpolation between the bottom vector and the top vector, depending on the weight calculated according to the Y position. This means when weight is 0, I’ll get the value as bottom_vector[x]; when weight is 1, I’ll get the values as top_vector [x]; and for values in between, I’ll get a blend. By doing this I get smooth transitions from the base to the top as I move vertically.
This is done in two regions- one between tbase and tmaxFv and the other one between tmaxFv and ttop.

- In each region I calculate:

- weight = (y - 0) / (h_maxFv-0)

- Because (by definition) the shortest temperature vector is longer than fbase (the longest Fv vector) and because we are interested in the Fv contour, we use the following 2 cases:

  - If x is within the range of fbase vector:
Interpolate between bottom_vector[x] and  top_vector [x] using the formula:
(1 - weight) * bottom_vector[x] + weight * top_vector[x]

  - If x is outside the range of fbase vector (so top_vector[x] does not exist), and the current matrix value is 300:
Interpolate between top[x] and a constant value of 300, using the same weight:
matrix[y, x] = (1 - weight) * 300 + weight * top_vector [x].item()   
Flame T is populated according to Flame Fv contour. In blue- T contour

# Generate RGB Image

## sootCalculation.mat

At this point I have 2 matrices for T and Fv.

Next, I want to generate the flame RGB. Generating the flame RGB is done using Victor’s SootImage tool.
The tool gets as an input a sootCalculation.mat file that holds in it 4 matrices: T, fv, r, z.

fv is fvmatrix, saved as is.

T is tmatrix clipped to fvmarix shape: 
tmatrix = tmatrix[:fvmatrix.shape[0], :fvmatrix.shape[1]]

r is a vector, size as the smaller side of the matrices. Values are from 0 in steps of 0.0662 (pixel size).

z is a vector, size as the larger side of the matrices. Values are from 0 in steps of 0.0662 (pixel size).

## Run through SootImage

Using sootImage tool (MATLAB files) with sootCalculation.mat as an input, I get the following files:
CFDImage.mat- holding the flame’s RGB image matrix
redMatrixCFD.mat- holding the flame’s R image matrix
greenMatrixCFD.mat- holding the flame’s G image matrix
blueMatrixCFD.mat- holding the flame’s B image matrix

Last thing I do is to generate an image out of CFDImage.mat and save an info.txt about the run.

# Results

For the DL model, I need data from:

- Input- RGB flame image

- Output- Soot Fraction Volume in every pixel of the flame

- Output- Temperature in every pixel of the flame

Using the processes and algorithms described above, I can create a dataset without the costly resources of a simulator.

Generating a sample takes approximately 30 seconds, varying in correspondence with the drawn size of the flame and times generated until conditions are met.

The following files are generated by the sample:

| File name | Description |
| --- | --- |
| info.txt | Summary of run, holding info such as h_maxFv and h_top, Fv and T Data (generated vectore, sizes, shapes, contours) and r, z shapes |
| ttop.csv | Top of flame’s temperature vector generated using Matlab function generateT(7) |
| tbase.csv | Base of flame’s temperature vector generated using Matlab function generateT(1) |
| tmaxFv.csv | Temperature vector at the height of the maximum soot fraction volume (h_maxFv) in the flame, generated using Matlab function generateT(4) |
| T_interpolated_matrix.csv | Matrix holding all values of flame’s temperature in Fv contour (cut to size of Fv matrix) |
| ftop.csv | Top of flame’s soot fraction volume vector generated using Matlab function generatef(7) |
| fbase.csv | Base of flame’s soot fraction volume vector generated using Matlab function generatef(1) |
| fmaxFv.csv | Soot fraction volume vector at the height of the maximum soot fraction volume (h_maxFv) in the flame, generated using Matlab function generatef(4) |
| Fv_interpolated_matrix.csv | Matrix holding all values of flame’s soot fraction volume |
| CFDImage.mat | MAT file holding flame RGB image 3D matrix |
| sootCalculation.mat | Mat file holding Fv, T, r, z matrices. This object is the input of sootImage tool and output of DL model |
| redMatrixCFD.mat | Mat file holding red matrix of the RGB flame image |
| greenMatrixCFD.mat | Mat file holding green matrix of the RGB flame image |
| blueMatrixCFD.mat | Mat file holding blue matrix of the RGB flame image |
| T_flame_plot_noContour.png | Plot of flame’s temperature values in each pixel (in Kelvin scale, ranging from 300 and up) |
| T_flame_plot_Contour.png | Plot of flame’s temperature values in each pixel (in Kelvin scale, ranging from 300 and up) with the contour according to the generated base, maxFv and top vectors (Note: the contour is according to the T vectors, The map is after cutting to Fv size) |
| Fv_flame_plot_noContour.png | Plot of flame’s soot fraction volume values in each pixel (in ppm scale, ranging from 0 and up) |
| Fv_flame_plot_Contour.png | Plot of flame’s soot fraction volume values in each pixel (in ppm scale, ranging from 0 and up) with the contour according to the generated base, maxFv and top vectors. |
| flame_CFDImage_image.png | RGB flame image |

Not all files are needed for our deep learning model. I saved data I thought might help us investigate results in the future.
The files we need for our deep learning model are:

- Input- CFDImage.mat

- Output- sootCalculation.mat (or T_interpolated_matrix.csv and Fv_interpolated_matrix.csv)

# Known Issues and Future Work

- The flame is not generated up to its tip, but up to a random height between 400-800.

- The mathematical relation between T and Fv is not considered.

- Add that if there is a temperature value higher than 2375- loose sample.

# Appendix A – Accepting Data Restrictions

When running the generatef.m and generateT.m functions, we are dealing with some randomicity of data. To provide data physically and mathematically as accurate as possible, a few restrictions and conditions were defined:

- h_maxFv, h_top, fbase, fmaxFv, ftop, tbase, tmaxFv, ttop set was not used before.

- len(fbase)>len(fmaxFv)>len(ftop).

- The max value of fmaxFV is greater than max value of fbase and of ftop.

- All T vectors’ length must be greater than fbase length.

* There is no restriction as to which vector is longer between the T vectors.

# Appendix B – Generate Data (Matlab Code)

## Generate Fv

## Generate T

# Appendix C – SootImage (Matlab Code)